SCHOOL NETWORK DESIGN HOME LAB

USING CISCO PACKET TRACER

Network Design Home Lab

Date: October 4, 2023

Overview

This report details the network design for a home lab with the given network address 10.3.99.0 The network is divided into 6 subnets as follows:

| Subnet Name | Subnet Mask | Subnet Address | Default Gateway | Broadcast Address | Usable Host Range |
| --- | --- | --- | --- | --- | --- |
| Admissions | 255.255.255.224 | 10.3.99.0 | 10.3.99.1 | 10.3.99.31 | 10.3.99.2 - 10.3.99.30 |
| Administration | 255.255.255.224 | 10.3.99.32 | 10.3.99.33 | 10.3.99.63 | 10.3.99.34 - 10.3.99.62 |
| Faculty | 255.255.255.224 | 10.3.99.64 | 10.3.99.65 | 10.3.99.95 | 10.3.99.66 - 10.3.99.94 |
| Student | 255.255.255.224 | 10.3.99.96 | 10.3.99.97 | 10.3.99.127 | 10.3.99.98 - 10.3.99.126 |
| Housing | 255.255.255.224 | 10.3.99.128 | 10.3.99.129 | 10.3.99.159 | 10.3.99.130 - 10.3.99.158 |
| Library | 255.255.255.224 | 10.3.99.160 | 10.3.99.161 | 10.3.99.191 | 10.3.99.162 - 10.3.99.190 |

Subnet Designs

The Administration and Housing subnets each contain 2 computers and 1 DHCP server.

The Admission contain  2 computers and 2 servers (DHCP and FTP)

The Faculty subnet contains 1 server, 2 laptops, and 4 computers.

The Student subnet contains 1 computer, 1 DHCP server supporting up to 20 devices, and 10 laptops.

The Library subnet contains 2 computers, 1 wireless router with 4 wireless devices.

All servers are assigned the last usable IP address in each subnet. All computers are assigned the first usable IP address. The wireless router is assigned the last usable IP address.

ACL Implementation

The ACLs were enabled and tested by pinging from a wireless device in the Library subnet to verify connectivity, then lack of connectivity when ACLs enabled, and restored connectivity when ACLs disabled.

4 ACLs were created:

- ACL 20 blocks Admissions servers              - ACL 30 blocks Administration servers

- ACL 40 blocks Faculty servers                        - ACL 50 blocks Student servers

The network was designed and configured according to the provided specifications. Connectivity testing validated the configuration and ACL implementations. Further optimization or changes to the design can be implemented as required.

Network Topology Diagram

[packet tracer topology diagram screenshot]

Device Configurations

Admissions Subnet:

Server (FTP): 10.3.99.29

Server (DHCP): 10.3.99.30

Computer 1: 10.3.99.2

Computer 2: 10.3.99.3

Administration Subnet:

Server (DHCP): 10.3.99.62

Computer 1: 10.3.99.34

Computer 2: 10.3.99.35

Faculty Subnet:

Server: 10.3.99.94

Laptop 1: 10.3.99.67

Laptop 2: 10.3.99.68

Computer 1: 10.3.99.66

Computer 2: 10.3.99.69

Computer 3: 10.3.99.70

Computer 4: 10.3.99.71

Student Subnet:

Server (DHCP): 10.3.99.126

Computer: 10.3.99.107

Laptop 1: 10.3.99.98 | Laptop 6: 10.3.99.103

Laptop 2: 10.3.99.99 | Laptop 7: 10.3.99.104

Laptop 3: 10.3.99.100 | Laptop 8: 10.3.99.105

Laptop 4: 10.3.99.101 | Laptop 9: 10.3.99.106

Laptop 5: 10.3.99.102 | Laptop 10: 10.3.99.108

Library Subnet:                                                 Housing Subnet:

Computer 1: 10.3.99.162                                  Server (DHCP): 10.3.99.158

Computer 2: 10.3.99.163                                  Computer 1: 10.3.99.130

Wireless Router: 10.3.99.190                           Computer 2: 10.3.99.131

Wireless Device 1: DHCP assigned.

Wireless Device 2: DHCP assigned.

Wireless Device 3: DHCP assigned.

Wireless Device 4: DHCP assigned.

CONNECTION TEST

PINGING EACH DEVEICES